PROJECT REPORT

Smart Plant Monitoring System

ESP32-Powered IoT Plant Care & Automation System

| Author | Ninad Hazarika |
| --- | --- |
| Email | ninadhazarika@gmail.com |
| IDE | Arduino IDE 2.x |
| Language | C++ (Arduino Framework) |
| Version | v2.0 — 2026 |
| License | MIT Open Source |

https://github.com/Ninad-Hazarika/smart-plant-monitoring-system

# 1.  Project Title & Introduction

## 1.1  Project Title

Smart Plant Monitoring System using ESP32

## 1.2  Introduction

Modern agriculture and home gardening face a persistent challenge: plants are frequently over-watered, under-watered, or neglected due to busy schedules and the absence of objective feedback. Traditional plant care relies entirely on human observation — a method prone to error, inconsistency, and forgetfulness.

This project presents a fully autonomous, IoT-enabled Smart Plant Monitoring System built on the ESP32 microcontroller. The system continuously measures soil moisture, ambient temperature and humidity, and water tank level. It drives a relay-controlled irrigation pump automatically based on configurable soil dryness thresholds, hosts a live dark-theme web dashboard accessible from any browser on the local network, and fires real-time push alerts directly to the user's Telegram account for five distinct critical events.

Beyond pure telemetry, the system incorporates a personality layer: an OLED display renders animated facial expressions that react to environmental conditions and human presence — greeting visitors through IR detection, expressing happiness when watered, sadness when the tank runs low, and playful laughter when a triple-touch gesture is detected.

Key Highlights at a Glance: • 9 peripherals fused on a single ESP32 — sensors, actuators, display, network • Three pump modes: AUTO (soil-threshold) / MANUAL ON / MANUAL OFF • 5-state animated OLED emotion engine • Real-time Telegram bot push alerts over HTTPS • Local Wi-Fi web dashboard — no cloud required

# 2.  Problem Statement

The following core problems motivate the design of this system:

- Inconsistent manual watering causes root rot (over-watering) or wilting (under-watering), both of which cause lasting damage to plant health.

- The absence of real-time sensor feedback means the user has no objective data to inform watering decisions.

- Water tank depletion goes undetected — automated pump systems run dry, damaging the pump and failing to irrigate the plant.

- No alert mechanism means problems are only discovered after visible damage has already occurred.

- Human presence near sensitive plants goes unlogged, which matters in shared spaces or research environments.

The proposed solution addresses all five problems through sensor fusion, rule-based automation, and real-time communication — requiring zero manual intervention under normal operating conditions.

# 3.  Components Used

## 3.1  Bill of Materials

| Component | Specification | Qty | GPIO / Interface | Purpose |
| --- | --- | --- | --- | --- |
| ESP32 DevKit v1 | Dual-core LX6, 240 MHz, Wi-Fi + BT | 1 | — | Main MCU + network |
| DHT11 Sensor | 0-50 C, 20-90% RH, digital 1-wire | 1 | GPIO 13 | Temp & humidity |
| Soil Moisture Sensor | Capacitive, analog, 3.3V | 1 | GPIO 34 (ADC) | Soil wetness 0-4095 |
| HC-SR04 Ultrasonic | 2-400 cm, +/-3 mm, 5V | 1 | TRIG:26 ECHO:25 | Water tank level |
| OLED SSD1306 | 0.96", 128x64 px, I2C, 3.3V | 1 | SDA:21 SCL:22 | Live display + face |
| 5V Relay Module | Single ch., active LOW, 10A/250VAC | 1 | GPIO 15 | Pump switching |
| Mini DC Water Pump | 3-6V, submersible, ~100 L/h | 1 | Via relay | Irrigation |
| IR Obstacle Sensor | Digital OUT, adjustable detection range | 1 | GPIO 33 | Human detection |
| Touch Sensor (TTP223) | Capacitive, digital OUT, 3.3-5V | 1 | GPIO 32 | Gesture input |
| Passive Buzzer | 5V, ~2 kHz | 1 | GPIO 27 | Low-water alert |
| 9V Adapter / Battery | Isolated pump supply | 1 | — | Pump power |
| USB Micro Cable | Programming + ESP32 power | 1 | — | Development |
| Jumper Wires | M-M / M-F assorted | ~30 | — | Wiring |
| Breadboard / PCB | 830-point or custom EasyEDA PCB | 1 | — | Assembly |

## 3.2  Library Dependencies

| Library | Author | Purpose |
| --- | --- | --- |
| Adafruit GFX Library | Adafruit | OLED graphics primitives (circles, lines, text) |
| Adafruit SSD1306 | Adafruit | SSD1306 OLED driver over I2C |
| DHT sensor library | Adafruit | DHT11 / DHT22 one-wire protocol driver |
| Adafruit Unified Sensor | Adafruit | Unified hardware abstraction layer |
| WiFi (built-in) | Espressif | Wi-Fi station mode and connection management |
| WebServer (built-in) | Espressif | HTTP server on port 80 for web dashboard |
| HTTPClient (built-in) | Espressif | HTTPS GET client for Telegram Bot API |
| WiFiClientSecure (built-in) | Espressif | TLS/SSL transport for Telegram HTTPS calls |

# 4.  Working Explanation

## 4.1  System Boot Sequence

On power-up the ESP32 initialises all GPIO pins, starts the DHT11 driver, initialises the OLED display and shows a startup message, connects to Wi-Fi, synchronises time via NTP (pool.ntp.org), starts the HTTP web server on port 80, and transmits a 'System Online' Telegram notification. The full boot sequence completes in approximately 5-7 seconds.

## 4.2  Main Execution Loop

The main loop executes with a 50 ms base delay. Work is divided into per-loop and per-second tasks:

### Every iteration (50 ms):

- server.handleClient() — serves incoming HTTP requests from browsers

- handleIR() — polls GPIO 33; detects human presence on LOW signal; triggers GREETING emotion on rising edge

- handleTouch() — polls GPIO 32; tracks rapid triple-tap within 900 ms window to trigger LAUGHING emotion

### Every 1000 ms (sensor cycle):

- readSoil() — 12-bit ADC read on GPIO 34 (higher value = drier soil)

- readDHT() — reads temperature and humidity from DHT11 with NaN protection

- readWaterLevel() — sends 10 µs TRIG pulse; measures ECHO duration; computes: waterLevel = tankHeight - (duration x 0.034 / 2)

- controlPump() — evaluates pumpMode and drives relay accordingly; triggers HAPPY on pump start

- checkTankLevel() — pulses buzzer on GPIO 27 if waterLevel is at or below 3 cm

- updateOLED() — renders active emotion face or sensor data screen

- checkTelegramAlerts() — evaluates all five alert conditions with independent cooldown timers

## 4.3  Pump Control Modes

| Mode | pumpMode | Behaviour | Emotion |
| --- | --- | --- | --- |
| AUTO | 0 | Pump ON when soilValue > threshold; OFF when moist again | HAPPY when pump first turns ON |
| MANUAL ON | 1 | Pump forced ON regardless of soil moisture reading | HAPPY immediately |
| MANUAL OFF | 2 | Pump forced OFF regardless of soil moisture reading | None |

## 4.4  OLED Emotion Engine

The emotion engine uses a priority system: when currentEmotion is not NONE, the OLED renders the face animation. When emotionUntil is exceeded (millis() > emotionUntil), the system returns to NONE and the sensor data screen is shown. Emotion drawing uses Adafruit GFX primitives on the 128x64 pixel canvas.

| Emotion | Trigger | Duration | OLED Text |
| --- | --- | --- | --- |
| GREETING | IR sensor detects object (rising edge on GPIO 33) | 3 seconds | Hello! |
| HAPPY | Pump turns ON (auto threshold or manual) | 3 seconds | Ahh... Water! |
| SAD | Water level at or below 3 cm (repeats every 10s) | 3s per cycle | I'm thirsty |
| LAUGHING | 3 touch events within 900 ms on GPIO 32 | 5 seconds | Hehehe! |
| SLEEPY | No IR trigger for 60+ consecutive seconds | 3 seconds | Zzz... |
| NONE | Any emotion timer expires (emotionUntil exceeded) | Until next event | [Sensor Data Screen] |

## 4.5  Telegram Alert System

Alerts are delivered using WiFiClientSecure with setInsecure() to establish TLS sessions with api.telegram.org without certificate pinning. All message text is URL-encoded before embedding in the GET request URL. Five independent alert conditions are checked on every sensor cycle:

| Alert | Trigger Condition | Cooldown | Auto-Reset When |
| --- | --- | --- | --- |
| System Online | Boot completes | Once | N/A |
| Low Water | waterLevel at or below 3 cm | Per event | waterLevel > 5 cm |
| Pump Overtime | Pump ON for more than 60 continuous seconds | Per event | Pump turns OFF |
| Plant Thirsty | soilValue > threshold AND pump OFF for 60s+ | Per event | Moist OR pump ON |
| Human Detected | IR LOW for 2+ consecutive loop polls | 5 seconds | IR returns HIGH |

## 4.6  Web Dashboard

The ESP32 hosts a single-page dark-theme dashboard on port 80. Sensor values are embedded server-side at render time. The page auto-refreshes every 2 seconds via JavaScript setTimeout(). Pump mode changes are handled as HTTP 303 redirects. The dashboard is fully responsive and works on any mobile or desktop browser on the same Wi-Fi network.

# 5.  Circuit Diagram

## 5.1  Complete GPIO Pin Assignment

| Peripheral | Peripheral Pin | ESP32 Pin | Signal | Notes |
| --- | --- | --- | --- | --- |
| DHT11 | Data OUT | GPIO 13 | Digital 1-wire | Add 10k pull-up if no onboard resistor |
| DHT11 | VCC | 3.3V | Power |  |
| DHT11 | GND | GND | Ground |  |
| Soil Sensor | AO | GPIO 34 | Analog ADC | Input-only pin; 12-bit resolution |
| Soil Sensor | VCC | 3.3V | Power |  |
| HC-SR04 | TRIG | GPIO 26 | Digital OUT | 10 µs HIGH pulse to trigger |
| HC-SR04 | ECHO | GPIO 25 | Digital IN | Use voltage divider if module outputs 5V |
| OLED SSD1306 | SDA | GPIO 21 | I2C Data | I2C address 0x3C |
| OLED SSD1306 | SCL | GPIO 22 | I2C Clock |  |
| OLED SSD1306 | VCC | 3.3V | Power |  |
| Relay Module | IN | GPIO 15 | Digital OUT | Active LOW — RELAY_ON = LOW |
| Relay Module | VCC | VIN (5V) | Power | Must use 5V, not 3.3V |
| IR Sensor | OUT | GPIO 33 | Digital IN | LOW = object/human detected |
| Touch Sensor | OUT | GPIO 32 | Digital IN | HIGH = touch detected |
| Buzzer | Positive | GPIO 27 | Digital OUT | HIGH = buzzer sounds |

## 5.2  Power & Safety Notes

- CRITICAL: Never connect the water pump directly to any ESP32 GPIO pin. The relay provides full galvanic isolation between the 3.3V control circuit and the 9V pump circuit.

- Connect the negative terminal (GND) of the 9V pump supply to the ESP32 GND to create a common ground reference.

- GPIO 34 is an ADC input-only pin — do not attempt to configure it as OUTPUT.

- The HC-SR04 ECHO pin on some modules outputs 5V logic. Use a 1k/2k voltage divider before GPIO 25 to avoid damaging the ESP32.

Schematic & PCB files: • EasyEDA schematic (.json, .svg) — available in /schematics/ on GitHub • PCB layout and Gerber files for fabrication — available in /pcb/ on GitHub • PCB can be ordered from JLCPCB for approximately $2 USD per 5 boards

# 6.  Source Code

## 6.1  Repository File Structure

| File / Folder | Description |
| --- | --- |
| src/smart_plant_monitoring.ino | Main firmware — complete system with all 9 peripherals |
| src/pump_relay_test.ino | Isolated relay and pump test (toggles every 3 seconds) |
| src/sensor_test.ino | DHT11 + soil sensor diagnostic (Serial Monitor output) |
| hardware/component_list.md | Full BOM with specifications and sourcing information |
| hardware/wiring_guide.md | Step-by-step connection guide for each peripheral |
| schematics/ | EasyEDA circuit schematic files (.json, .svg) |
| pcb/ | PCB layout and Gerber files for board fabrication |
| docs/ | Project report (this document) and technical notes |
| website/index.html | Standalone web dashboard UI reference file |
| README.md | Full technical documentation and quick-start guide |

## 6.2  Key Code Sections

### Water Level Measurement — HC-SR04

void readWaterLevel() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH); // trigger 10us pulse

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 30000); // 30ms timeout

if (duration == 0) { waterLevel = -1; return; } // no echo = error

float distance = duration * 0.034 / 2; // convert to cm

waterLevel = tankHeight - distance; // invert: less distance = more water

if (waterLevel < 0) waterLevel = 0;

}

### Three-Mode Pump Control

void controlPump() {

bool oldState = pumpState;

if (pumpMode == 1) { digitalWrite(RELAY_PIN, RELAY_ON); pumpState = true; }

else if (pumpMode == 2) { digitalWrite(RELAY_PIN, RELAY_OFF); pumpState = false; }

else { // AUTO — soil threshold control

if (soilValue > threshold) { digitalWrite(RELAY_PIN, RELAY_ON); pumpState = true; }

else { digitalWrite(RELAY_PIN, RELAY_OFF); pumpState = false; }

}

if (!oldState && pumpState) { // pump just turned ON

currentEmotion = HAPPY;

emotionUntil = millis() + 3000;

}

}

### Triple-Touch Gesture Detection

void handleTouch() {

bool touchState = digitalRead(TOUCH_PIN);

if (touchState == HIGH && lastTouchState == LOW) { // rising edge only

if (millis() - lastTouchTime < 900) touchCount++; // within 900ms window

else touchCount = 1; // too slow, restart count

lastTouchTime = millis();

if (touchCount >= 3) { // triple tap!

currentEmotion = LAUGHING;

emotionUntil = millis() + 5000;

touchCount = 0;

}

}

lastTouchState = touchState;

}

### Telegram HTTPS Push Alert

void sendTelegramMessage(String message) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure client;

client.setInsecure(); // TLS without cert pinning (local IoT)

HTTPClient https;

https.setTimeout(15000);

String url = "https://api.telegram.org/bot" + botToken +

"/sendMessage?chat_id=" + chatID +

"&text=" + urlEncode(message);

https.begin(client, url);

int code = https.GET();

https.end();

}

Full source code is available at:

https://github.com/Ninad-Hazarika/smart-plant-monitoring-system

# 7.  Testing Results

## 7.1  Individual Component Test Results

| Test Case | Method | Expected Outcome | Result |
| --- | --- | --- | --- |
| Relay cycle test | pump_relay_test.ino — 3s ON / 3s OFF loop | Relay clicks audibly, pump runs and stops | PASS |
| DHT11 temperature | Compare with calibrated reference thermometer | Within +/-2 degrees C | PASS |
| DHT11 humidity | Compare with calibrated reference hygrometer | Within +/-5% RH | PASS |
| Soil sensor dry | Sensor in open air | ADC reads 3800-4095 | PASS |
| Soil sensor wet | Probe submerged in water | ADC reads 0-600 | PASS |
| HC-SR04 level read | Known 5 cm water depth in tank | Reported 5.0 +/- 0.5 cm | PASS |
| OLED display init | Boot sequence, I2C address scan | Display initialises, shows startup message | PASS |
| Wi-Fi connection | Serial Monitor at 115200 baud on boot | IP address assigned, dashboard accessible | PASS |
| Telegram bot online alert | Boot sequence | Message received on Telegram within 10s | PASS |
| IR human detection | Wave hand in front of sensor | GREETING emotion + Telegram ping fired | PASS |
| Triple-touch gesture | Tap sensor 3 times within 900ms | LAUGHING emotion triggered for 5 seconds | PASS |
| Buzzer low water | Set water level to 2 cm during test | Buzzer beeps each sensor cycle | PASS |
| AUTO pump ON (dry soil) | Cover sensor with dry material above threshold | Pump activates within 1 second | PASS |
| AUTO pump OFF (wet soil) | Submerge sensor while pump running | Pump stops within 1 second | PASS |
| Web dashboard access | Open IP in Chrome on mobile and desktop | Dashboard loads, sensors shown, auto-refreshes | PASS |
| Manual pump ON via web | Click ON button in dashboard | Pump activates, pumpMode set to 1 | PASS |
| Manual pump OFF via web | Click OFF button in dashboard | Pump stops, pumpMode set to 2 | PASS |
| AUTO mode via web | Click AUTO button in dashboard | Returns to soil-threshold control | PASS |

## 7.2  Integrated System Test

A full 30-minute continuous operation test was conducted with all components connected simultaneously. Key observations:

- Sensor readings updated reliably every 1 second with no missed cycles across the test period

- Web dashboard remained accessible throughout with auto-refresh functioning correctly

- Pump cycled ON and OFF correctly in AUTO mode as soil moisture changed over the watering cycle

- All five OLED emotion states triggered correctly in their respective scenarios

- Telegram alerts fired for low water and dry soil conditions with correct cooldown behaviour

- No Wi-Fi disconnections or system crashes observed during the 30-minute test window

- Serial Monitor output confirmed correct sensor values and pump state at all times

# 8.  Challenges Faced

| Challenge | Description | Solution |
| --- | --- | --- |
| DHT11 NaN reads | DHT11 returns NaN on first read after boot and intermittently under electrical noise. | Added isnan() guard before assigning values; the last valid reading is retained if a new read fails. |
| HC-SR04 infinite block | Without a timeout, pulseIn() blocks the entire loop indefinitely when the tank is empty and no echo returns. | Added 30000 µs (30ms) timeout parameter; waterLevel is set to -1 on timeout and the dashboard shows 'No Echo'. |
| Relay floating at boot | GPIO 15 floats LOW briefly at boot, causing the relay to activate and the pump to turn ON unexpectedly. | Added explicit digitalWrite(RELAY_PIN, RELAY_OFF) as the very first action in setup() before any other initialisation. |
| Telegram TLS failure | ESP32 HTTPClient with full TLS certificate verification fails against api.telegram.org due to missing root CA store. | Used WiFiClientSecure with setInsecure() which skips certificate verification — acceptable for a trusted local IoT device. |
| HTML string memory spike | Building one large HTML string caused heap fragmentation and occasional crashes on early firmware builds. | Refactored to R"rawliteral()" raw string literals and split generation into smaller concatenated chunks to reduce peak RAM allocation. |
| OLED display flickering | Competing sensor data and emotion display code both called display.clearDisplay() in the same loop, causing visible flicker. | Introduced the emotionUntil timer system; emotion display takes exclusive priority and sensor data only renders when currentEmotion == NONE. |
| Soil sensor calibration | ADC values varied significantly between sensors of the same model and with different soil compositions and moisture levels. | Exposed threshold as a top-level configurable variable and documented a calibration procedure in the README. |

# 9.  Future Improvements

| Improvement | Priority | Description |
| --- | --- | --- |
| MQTT / Home Assistant | High | Publish sensor readings to an MQTT broker for integration with Home Assistant, Node-RED, or Grafana dashboards. |
| OTA Firmware Updates | High | Enable ArduinoOTA so the device can receive firmware updates over Wi-Fi without physical USB access. |
| Firebase Data Logging | Medium | Push timestamped readings to Firebase Realtime Database for long-term historical analysis and trend graphs. |
| Daily Telegram Summary | Medium | Send a daily summary at a scheduled time with min/max/average values for all sensors using NTP time. |
| SD Card CSV Logging | Medium | Log all sensor readings to microSD as CSV for offline analysis and resilience against Wi-Fi loss. |
| Custom PCB + Enclosure | Medium | Design a production-grade two-layer PCB in EasyEDA and a 3D-printed weatherproof enclosure. |
| Solar + Battery Backup | Low | Add LiPo battery with TP4056 charging and a small solar panel for fully off-grid autonomous operation. |
| Multi-Zone Irrigation | Low | Support multiple soil sensors and relays for independent control of several plant zones. |
| REST API Endpoint | Low | Expose a JSON API so Google Home, Alexa, or custom mobile apps can query sensor values and control the pump. |
| Voice Assistant | Low | Integrate with Google Home / Amazon Alexa via the REST API for voice-controlled irrigation commands. |

# 10.  Project Images, Demo Video & Submission Links

## 10.1  Project Images

All project photographs and screenshots are stored in the /images/ folder of the GitHub repository and in the Google Drive submission folder.

Images submitted include: • Full assembled circuit on breadboard • OLED emotion display — all 5 states (GREETING, HAPPY, SAD, LAUGHING, SLEEPY) • Web dashboard screenshot on mobile and desktop browsers • Telegram bot alert notification screenshot • EasyEDA PCB layout render (top and bottom copper layers)

## 10.2  Demo Video

A full demonstration video showing the system operating in real time — including auto-watering triggering, all emotion transitions, the live web dashboard, and Telegram alerts firing — is available at:

Demo Video: https://drive.google.com/drive/folders/1ZfgRjcn0nLJQALMsFx7dsPL00-i2h-ok?usp=sharing

## 10.3  Submission Links

| Deliverable | Link |
| --- | --- |
| GitHub Repository | https://github.com/Ninad-Hazarika/smart-plant-monitoring-system |
| Google Drive Folder | https://drive.google.com/drive/folders/1ZfgRjcn0nLJQALMsFx7dsPL00-i2h-ok?usp=sharing |
| LinkedIn | www.linkedin.com/in/ninad-hazarika-14a27937a |

## 10.4  References

- Espressif Systems. ESP32 Technical Reference Manual. https://docs.espressif.com/

- Adafruit Industries. Adafruit SSD1306 Library. https://github.com/adafruit/Adafruit_SSD1306

- Adafruit Industries. DHT Sensor Library. https://github.com/adafruit/DHT-sensor-library

- Telegram. Bot API Documentation. https://core.telegram.org/bots/api

- Arduino. Language Reference. https://www.arduino.cc/reference/en/

- EasyEDA. Online PCB Design Tool. https://easyeda.com/

- JLCPCB. PCB Fabrication Service. https://jlcpcb.com/

# 11.  PCB Design & CAD Files

## 11.1  PCB Design

The printed circuit board was designed using EasyEDA (https://easyeda.com/), a free browser-based PCB design tool with direct one-click ordering integration for JLCPCB.

- PCB project files (.json) are located in /pcb/ on the GitHub repository

- Gerber files for fabrication are packaged as /pcb/gerber.zip

- The board consolidates all sensor headers, relay footprint, OLED header, and ESP32 DevKit socket on a single compact board

- Can be fabricated at JLCPCB for approximately $2 USD per 5-board order

- Minimum trace width 0.3 mm; two copper layers; HAL surface finish

## 11.2  CAD / Enclosure Design

A 3D-printed weatherproof enclosure is planned for v3.0. The current prototype uses an open breadboard assembly. CAD files (Fusion 360 / FreeCAD .f3d / .FCStd) will be added to the /cad/ folder in the next release.

## 11.3  Schematic Files

EasyEDA circuit schematics (.json native format and exported .svg) are located in the /schematics/ folder. The schematic covers all nine peripheral connections to the ESP32 DevKit including power rail annotations, GPIO labels, and component values.

Project Summary

| Item | Detail |
| --- | --- |
| Project Name | Smart Plant Monitoring System |
| Author | Ninad Hazarika — ninadhazarika@gmail.com |
| MCU | ESP32 DevKit v1 (Dual-core LX6, 240 MHz, Wi-Fi + BT) |
| Sensors | DHT11, Capacitive Soil, HC-SR04 Ultrasonic, IR, Touch |
| Actuators | 5V Relay, Mini DC Water Pump, Passive Buzzer |
| Display | OLED SSD1306 — 0.96", 128x64, I2C @ 0x3C |
| Connectivity | Wi-Fi 802.11 b/g/n — local HTTP server port 80 |
| Notifications | Telegram Bot — HTTPS push (5 distinct alert types) |
| Pump Modes | AUTO (soil threshold) / MANUAL ON / MANUAL OFF |
| Emotion Engine | 5 states: GREETING / HAPPY / SAD / LAUGHING / SLEEPY |
| Web Dashboard | Responsive dark-theme UI, 2-second auto-refresh |
| Alert Cooldowns | 30s global Telegram cooldown; 5s for human detection |
| Language | C++ (Arduino Framework) + HTML/CSS/JS (web UI) |
| License | MIT Open Source |
| GitHub | github.com/ninadhazarika/smart-plant-monitoring-system |

"Plants can't talk — so we built something that does."